微算機系統

實驗二

組別： 108590451

班級、姓名與學號： 資工二 陸詠涵 108580451

日期： 2020.10.08

- 實驗內容：

本次實驗主旨目標

將上課新教的package語法、之前實驗一(七段顯示器)和全加法器

結合，實際了解組合電路的內部運作，及最後燒錄的結果。

- 目標一-8-bits 多位元加法器

- 目標二-BCD加法器

- 實驗過程及結果：

撰寫你如何完成本次作業的流程方法。

請附上結果圖片(放置於此部分即可)。

- 目標一-8-bits 多位元加法器:

實驗步驟:

Step1:設計VHDL程式碼

利用上課所學的package 語法，分別寫出全加法器及七段顯示器(實驗一)的程式，再寫8-bits多元加法器的主程式來連接(利用USE work.fulladd_package.all ;

和USE work.seven_segment_display_package.all ;先將全加法器及七段顯示器呼叫主程式，再利用stage0: fulladd PORT MAP ( '0', X(0), Y(0), S(0), CC(1) ) ;來對應實際的inputs和outputs)。

(picture 1)七段顯示器的VHDL程式碼

(picture 2)七段顯示器packag的VHDL程式碼

(picture 3)全加法器的VHDL程式碼

(picture 4)全加法器package的VHDL程式碼

(picture 5)8-bits 多位元加法器的VHDL主程式碼

Step2:接腳位

(picture 6) 8-bits 多位元加法器邏輯電路圖

(picture 7) 8-bits 多位元加法器所接的腳位

Step3:燒錄到模板中

(picture 8) 8-bits 多位元加法器3+3所加的結果

- 目標二-BCD加法器

實驗步驟:

Step1:設計VHDL程式碼

將目標一的fulladder和七段顯示器的程式碼留著，再另外寫一個BCDadder的主程式碼(大部分和8-bits 多位元加法器的VHDL主程式碼，但因BCDadder是利用4-bits顯示0~9的是數字，再加法上會有進位的問題 ex:當num >10時，要進位”加110(base 2)”，所以多了carry <= ( S(3) and S(2) ) or ( S(3) and S(1) ) or CC(4) ; 來判斷進位)。

(picture 9)BCD加法器的VHDL主程式碼

Step2:接腳位

(picture 10) BCD加法器邏輯電路圖

(picture 11) BCD加法器所接的腳位

Step3:燒錄到模板中

(picture 12) BCD加法9+9的燒錄結果

- 程式碼（請調整成最小行高，行高0點）

目標一

(程式碼) 實驗二目標一 fulladd LIBRARY ieee ; USE ieee.std_logic_1164.all ; ENTITY fulladd IS PORT ( Cin, x, y : IN STD_LOGIC ; s, Cout : OUT STD_LOGIC ) ; END fulladd ; ARCHITECTURE LogicFunc OF fulladd IS BEGIN s <= x XOR y XOR Cin ; Cout <= (x AND y) OR (Cin AND x) OR (Cin AND y) ; END LogicFunc ; fulladd_package LIBRARY ieee ; USE ieee.std_logic_1164.all ; PACKAGE fulladd_package IS COMPONENT fulladd PORT ( Cin, x, y : IN STD_LOGIC ; s, Cout : OUT STD_LOGIC ) ; END COMPONENT ; END fulladd_package ; seven_segment_display Library ieee; USE ieee.std_logic_1164.all; ENTITY seven_segment_display IS PORT (W,X,Y,Z : IN STD_LOGIC; A,B,C,D,E,F,G : OUT STD_LOGIC); END seven_segment_display; ARCHITECTURE LogicFunc OF seven_segment_display IS BEGIN A <= (X AND NOT Y AND NOT Z) OR (W AND X AND NOT Y) OR (NOT W AND NOT X AND NOT Y AND Z) OR (W AND NOT X AND Y AND Z); B <= (X AND Y AND NOT Z) OR (W AND X AND Y) OR (W AND Y AND Z) OR (W AND X AND NOT Z) OR (NOT W AND X AND NOT Y AND Z); C <= (NOT W AND NOT X AND Y AND NOT Z) OR (W AND X AND NOT Z) OR (W AND X AND Y); D <= (NOT X AND NOT Y AND Z) OR (NOT W AND X AND NOT Y AND NOT Z) OR (X AND Y AND Z) OR (W AND NOT X AND Y AND NOT Z); E <= (NOT W AND Z) OR (NOT W AND X AND NOT Y) OR (NOT X AND NOT Y AND Z); F <= (NOT W AND NOT X AND Z) OR (NOT W AND NOT X AND Y) OR (NOT W AND Y AND Z) OR (W AND X AND NOT Y); G <= (NOT W AND NOT X AND NOT Y) OR (NOT W AND X AND Y AND Z); END LogicFunc; seven_segment_display_package LIBRARY ieee ; USE ieee.std_logic_1164.all ; PACKAGE seven_segment_display_package IS COMPONENT seven_segment_display PORT ( W,X,Y,Z : IN STD_LOGIC; A,B,C,D,E,F,G : OUT STD_LOGIC); END COMPONENT ; END seven_segment_display_package ; Adder8(8-bits) LIBRARY ieee ; USE ieee.std_logic_1164.all ; USE work.fulladd_package.all ; USE work.seven_segment_display_package.all ; USE ieee.std_logic_signed.all ; ENTITY adder8 IS PORT ( Cin : IN STD_LOGIC ; X, Y : IN STD_LOGIC_VECTOR(7 DOWNTO 0) ; Cout : OUT STD_LOGIC ; WW,XX,YY,ZZ: IN STD_LOGIC_VECTOR(1 DOWNTO 0) ; A,B,C,D,E,F,G : OUT STD_LOGIC_VECTOR(1 DOWNTO 0)) ; END adder8 ; ARCHITECTURE Structure OF adder8 IS SIGNAL CC : STD_LOGIC_VECTOR(1 TO 7) ; SIGNAL S : STD_LOGIC_VECTOR(7 DOWNTO 0) ; BEGIN stage0: fulladd PORT MAP ( '0', X(0), Y(0), S(0), CC(1) ) ; stage1: fulladd PORT MAP ( CC(1), X(1), Y(1), S(1), CC(2) ) ; stage2: fulladd PORT MAP ( CC(2), X(2), Y(2), S(2), CC(3) ) ; stage3: fulladd PORT MAP ( CC(3), X(3), Y(3), S(3), CC(4) ) ; stage4: fulladd PORT MAP ( CC(4), X(4), Y(4), S(4), CC(5) ) ; stage5: fulladd PORT MAP ( CC(5), X(5), Y(5), S(5), CC(6) ) ; stage6: fulladd PORT MAP ( CC(6), X(6), Y(6), S(6), CC(7) ) ; stage7: fulladd PORT MAP ( CC(7), X(7), Y(7), S(7), Cout ) ; stage8:seven_segment_display PORT MAP (s(7),s(6),s(5),s(4),A(1),B(1),C(1),D(1),E(1),F(1),G(1)); stage9:seven_segment_display PORT MAP (s(3),s(2),s(1),s(0),A(0),B(0),C(0),D(0),E(0),F(0),G(0)); END Structure ;

目標二

(程式碼) 實驗二目標二 Fulladd1 LIBRARY ieee ; USE ieee.std_logic_1164.all ; ENTITY fulladd IS PORT ( Cin, x, y : IN STD_LOGIC ; s, Cout : OUT STD_LOGIC ) ; END fulladd ; ARCHITECTURE LogicFunc OF fulladd IS BEGIN s <= x XOR y XOR Cin ; Cout <= (x AND y) OR (Cin AND x) OR (Cin AND y) ; END LogicFunc ; Fulladd1_package LIBRARY ieee ; USE ieee.std_logic_1164.all ; PACKAGE fulladd_package IS COMPONENT fulladd PORT ( Cin, x, y : IN STD_LOGIC ; s, Cout : OUT STD_LOGIC ) ; END COMPONENT ; END fulladd_package ; Seven1_segment_display Library ieee; USE ieee.std_logic_1164.all; ENTITY seven_segment_display IS PORT (W,X,Y,Z : IN STD_LOGIC; A,B,C,D,E,F,G : OUT STD_LOGIC); END seven_segment_display; ARCHITECTURE LogicFunc OF seven_segment_display IS BEGIN A <= (X AND NOT Y AND NOT Z) OR (W AND X AND NOT Y) OR (NOT W AND NOT X AND NOT Y AND Z) OR (W AND NOT X AND Y AND Z); B <= (X AND Y AND NOT Z) OR (W AND X AND Y) OR (W AND Y AND Z) OR (W AND X AND NOT Z) OR (NOT W AND X AND NOT Y AND Z); C <= (NOT W AND NOT X AND Y AND NOT Z) OR (W AND X AND NOT Z) OR (W AND X AND Y); D <= (NOT X AND NOT Y AND Z) OR (NOT W AND X AND NOT Y AND NOT Z) OR (X AND Y AND Z) OR (W AND NOT X AND Y AND NOT Z); E <= (NOT W AND Z) OR (NOT W AND X AND NOT Y) OR (NOT X AND NOT Y AND Z); F <= (NOT W AND NOT X AND Z) OR (NOT W AND NOT X AND Y) OR (NOT W AND Y AND Z) OR (W AND X AND NOT Y); G <= (NOT W AND NOT X AND NOT Y) OR (NOT W AND X AND Y AND Z); END LogicFunc; Seven1_segment_display_package LIBRARY ieee ; USE ieee.std_logic_1164.all ; PACKAGE seven_segment_display_package IS COMPONENT seven_segment_display PORT ( W,X,Y,Z : IN STD_LOGIC; A,B,C,D,E,F,G : OUT STD_LOGIC); END COMPONENT ; END seven_segment_display_package ; BCDadder LIBRARY ieee ; USE ieee.std_logic_1164.all ; USE work.fulladd_package.all ; USE work.seven_segment_display_package.all ; USE ieee.std_logic_signed.all ; ENTITY BCDadder IS PORT ( Cin : IN STD_LOGIC ; X, Y : IN STD_LOGIC_VECTOR(7 DOWNTO 0) ; Cout : OUT STD_LOGIC ; A,B,C,D,E,F,G : OUT STD_LOGIC_VECTOR(1 DOWNTO 0)) ; END BCDadder ; ARCHITECTURE a OF BCDadder IS SIGNAL CC : STD_LOGIC_VECTOR(8 downto 0) ; SIGNAL S : STD_LOGIC_VECTOR(3 DOWNTO 0) ; SIGNAL carry : STD_LOGIC ; SIGNAL temp : STD_LOGIC_VECTOR(3 DOWNTO 0) ; BEGIN stage0: fulladd PORT MAP ( '0', X(0), Y(0), S(0), CC(1) ) ; stage1: fulladd PORT MAP ( CC(1), X(1), Y(1), S(1), CC(2) ) ; stage2: fulladd PORT MAP ( CC(2), X(2), Y(2), S(2), CC(3) ) ; stage3: fulladd PORT MAP ( CC(3), X(3), Y(3), S(3), CC(4) ) ; carry <= ( S(3) and S(2) ) or ( S(3) and S(1) ) or CC(4) ; stage4: fulladd PORT MAP ('0', S(0), '0', temp(0), CC(5) ) ; stage5: fulladd PORT MAP ( CC(5), S(1), carry , temp(1), CC(6) ) ; stage6: fulladd PORT MAP ( CC(6), S(2), carry , temp(2), CC(7) ) ; stage7: fulladd PORT MAP ( CC(7), S(3), '0', temp(3), CC(8) ) ; stage8:seven_segment_display PORT MAP ('0','0','0',carry,A(1),B(1),C(1),D(1),E(1),F(1),G(1)); stage9:seven_segment_display PORT MAP (temp(3),temp(2),temp(1),temp(0),A(0),B(0),C(0),D(0),E(0),F(0),G(0)); END a;

- 實驗心得：

每個人的心得報告至少150字以上，有關於此實驗所遇到的難題，解決方法或是對於實驗過程的分析等。（每名組員都要寫）

這次實驗，我們組花了整整四節課的時間。前面第一、二節課，因為我們還不熟悉package的用法，所以只有分別把fulladder和七段顯示器的程式碼寫出來。之後，因為我們怕時間不夠，在期限內無法讓助教檢查，所以我們有利用回家的時間把fulladder、七段顯示器的package和8-bits 多位元加法器寫出來。所以在第三節課一上課十分鐘就把目標一搞定了。但是，BCDadder讓我和我的parnter卡觀很久，原因在於，我們兩個忘記BCD有進位的問題了。在找出問題點之後，我們嘗試了幾次之後，就成功了。