微算機系統

期末專案

組別： 16

班級、姓名與學號：

資工二 陸詠涵 108590451

資工二 陳佳均 108830035

日期：2021.01.14

- 實驗內容：

利用上課所學，設計出一個pipeline功能的簡易CPU。總實驗有四暫存器可用來儲存DATA,利用Rs和Rt來決定要存在哪個暫存器中，然後分別讓DATA,Rs和Rt接上七段顯示器，顯示在燒錄板子上。利用綠色LED來表示程式已執行到的步驟，再利用紅色LED來確保Hazard正在執行=>檢查Hazard是否發生。

- 實驗過程及結果：

我們要透過簡易CPU實現的功能有Load、Move、Add、Sub、And、Or、Nor、Slt

其中Add、Sub、And、Or、Nor、Slt需要測試是否有用到Hazard

<key>Hazard是一種用來偵測連續執行指令很重要的部分，將第一次執行的結果馬上write back然後再繼續執行指令。

這次pipeline的設計比較上一次的CPU多了四個variable，其功能是為了記錄每一階段所產生的結果。而且，為了解決Hazard的問題，我們將整個程式放入process中，並且利用倒敘的方式將variable依序往後放(EX:variable3=>variable4 再來是 variable2=>variable3)，如此便能不搞混順序。

Step1設計程式碼

(pipeline_CPU)

(主程式)

(seven segment)

Step2燒錄結果

- 程式碼

實驗

library ieee; use ieee.std_logic_1164.all; use ieee.numeric_std.all; entity PipelineCPU is port( Data: in std_logic_vector(7 downto 0); Instruction: in std_logic_vector(7 downto 0); --Instruction(7-4) : Opcode --Instruction(3-2) : Rs address --Instruction(1-0) : Rt address Clock: in std_logic; D_Rs: out std_logic_vector(7 downto 0); D_Rt: out std_logic_vector(7 downto 0); HazardDetection: out std_logic; ExcuteStatus: out std_logic_vector(3 downto 0)); end PipelineCPU; architecture PCPU_arch of PipelineCPU is function getOpcode(Ins: std_logic_vector(7 downto 0)) return std_logic_vector is begin return Ins(7 downto 4); end function; signal r0, r1, r2, r3: std_logic_vector(7 downto 0); begin D_Rs <= r0 when Instruction(3 downto 2) = "00" else r1 when Instruction(3 downto 2) = "01" else r2 when Instruction(3 downto 2) = "10" else r3; D_Rt <= r0 when Instruction(1 downto 0) = "00" else r1 when Instruction(1 downto 0) = "01" else r2 when Instruction(1 downto 0) = "10" else r3; process(Clock) variable tmpIns1: std_logic_vector(7 downto 0) := "11110000"; --Instruction fetch variable tmpIns2: std_logic_vector(7 downto 0) := "11110000"; --Instruction decode variable tmpIns3: std_logic_vector(7 downto 0) := "11110000"; --Excution variable tmpIns4: std_logic_vector(7 downto 0) := "11110000"; --Write back variable excuteResult: std_logic_vector(7 downto 0); variable tmpRs, tmpRt, tmpData1, tmpData2: std_logic_vector(7 downto 0); begin if rising_edge(Clock) then --Write back tmpIns4 := tmpIns3; if not (getOpcode(tmpIns4) = "1111") then ExcuteStatus(3) <= '1'; case tmpIns4(3 downto 2) is --Rs address when "00" => r0 <= excuteResult; when "01" => r1 <= excuteResult; when "10" => r2 <= excuteResult; when others => r3 <= excuteResult; end case; else ExcuteStatus(3) <= '0'; end if; --Excution tmpIns3 := tmpIns2; case getOpcode(tmpIns3) is when "0000" => --Load excuteStatus(2) <= '1'; excuteResult := tmpData2; when "0001" => --Move excuteStatus(2) <= '1'; excuteResult := tmpRt; when "0010" => --Add excuteStatus(2) <= '1'; excuteResult := std_logic_vector(signed(tmpRs) + signed(tmpRt)); when "0011" => --Sub excuteStatus(2) <= '1'; excuteResult := std_logic_vector(signed(tmpRs) - signed(tmpRt)); when "0100" => --And excuteStatus(2) <= '1'; excuteResult := tmpRs and tmpRt; when "0101" => --Or excuteStatus(2) <= '1'; excuteResult := tmpRs or tmpRt; when "0110" => --Nor excuteStatus(2) <= '1'; excuteResult := tmpRs nor tmpRt; when "0111" => --slt excuteStatus(2) <= '1'; if unsigned(tmpRs) < unsigned(tmpRt) then excuteResult := "00000001"; else excuteResult := "00000000"; end if; when others => --No ins excuteStatus(2) <= '0'; end case; --Instruction decode tmpIns2 := tmpIns1; if not (getOpcode(tmpIns2) = "1111") then excuteStatus(1) <= '1'; if (tmpIns3(3 downto 2) = tmpIns2(3 downto 2)) or (tmpIns3(3 downto 2) = tmpIns2(1 downto 0)) then HazardDetection <= '1'; tmpRs := excuteResult; else HazardDetection <= '0'; case tmpIns2(3 downto 2) is when "00" => tmpRs := r0; when "01" => tmpRs := r1; when "10" => tmpRs := r2; when others => tmpRs := r3; end case; end if; case tmpIns2(1 downto 0) is when "00" => tmpRt := r0; when "01" => tmpRt := r1; when "10" => tmpRt := r2; when others => tmpRt := r3; end case; tmpData2 := tmpData1; else HazardDetection <= '0'; excuteStatus(1) <= '0'; end if; --Instruction fetch if not (getOpcode(Instruction) = "1111") then excuteStatus(0) <= '1'; tmpData1 := Data; tmpIns1 := Instruction; else excuteStatus(0) <= '0'; tmpIns1 := "11110000"; end if; end if; end process; end PCPU_arch;

- 實驗心得：

陸詠涵：

本次實驗，因為和實驗七的執行步驟很像，所以我們利用實驗七的程式來改。我們的想法是，將4個步驟各自設立不同的process，再利用rising_edge clock來達成每按一次，執行到下一個步驟的結果。但是在設計的過程中，我們遇到了在不同的process重複使用到相同的變數，所以在compile的過程中，當然不會通過。然後還有Hazard的部分

，他需要先將上一個指令write back然後再繼續執行下一個指令。所以我們回去討論之後，發現可以將所以指令放在同一個process裡，這樣不但更快速，且不會因為時間差而有錯誤。最後，我們粗心的將腳位接錯了，所以debug了一陣子。謝謝老師和助教這學期的用心指導，隨然每次過程都卡卡的，但是期末的收穫真的很多。

陳佳均：

這次的期末實驗本來想用VHDL程式，也就是同時運行的思維來寫的。但是，當我們在設變數的時候卻發現有些變數的功能好像重複了(沒錯，就是設了一些沒意義的變數)，有些功能卻還需要其他變數，搞得我頭實在很痛。於是我們最後毅然決然地放棄了寫了一個禮拜的程式，重新在寫了一遍process的版本，也就是類似於python這種有順序的程式的思維方式。

再來就是最後在檢查的時候，真的沒想到會檢查那麼久，對不起助教了，不過還是好好笑，哈哈。因為我們怕沒檢查通過要留時間改進，所以我們程式一跑過就教助教了，完全沒發現我們的opcode跟rs,rt指標順序錯了，超糗的。

最後，感謝助教每一堂課的幫忙，謝謝!!